Selection of bio-based building envelope materials under the equivalent thermal performance assumption: how the framing determines the result

ÖNEMLİ: Sarı dolgulu yerleri BU DOSYA ÜZERİNDE DOLDURMAYINIZ. / İsim, Kurum Adresi (Türkçe ve İngilizce olarak verilmelidir), vd. bilgiler KAPAK SAYFASI hazırlanarak ayrı dosya olarak dergipark sisteme yüklenmelidir. (bknz Yazar Rehberi/Kapak Sayfası Taslak dosya)

| Author11*, Author21, Author32 |  |  |
| --- | --- | --- |
| 1Department, Faculty, University, City, Postal Code, Country 2Department, Faculty, University, City, Postal Code, Country |  |  |
| Highlights: |  | Graphical/Tabular Abstract |
| - A fixed thermal transmittance target alone yields no climate-zone ranking difference - A zone effect arises only through structures that break the proportionality - The life cycle system boundary changes the top-ranked material in every zone |  | Figure A summarises the three findings of this study in a single image. Panel (a) of Figure A shows the inter-zone ratio of the insulation thickness required to meet the recommended thermal transmittance of each degree-day zone; the bold line is the theoretical ratio and the grey lines are the realised ratios of the eighteen alternatives, which coincide with it. Because that ratio is identical for every material, the framing cannot produce a climate-zone effect. Panel (b) of Figure A gives the share of the total weight that each weighting method assigns to its single heaviest criterion: entropy concentrates about one third of the weight on one ordinal criterion, whereas CRITIC and equal weighting distribute it. Panel (c) of Figure A gives the end-of-life release fraction at which the advantage of bio-based materials disappears in each climate zone.ular abstract summarizes the contents of the article in a concise. Authors must provide images and data that clearly represent the work described in the article. The image should be readable using a regular screen resolution of 96 dpi - Figure A üzerindeki Figure A. Three framing effects on the ranking of bio-based building envelope materials Aşağıda belirtilen metin kısmındaki başlıklar (Purpose, Theory and Methods, Results, Conclusion) Purpose: Comparative studies of bio-based building envelope materials commonly adopt an equivalent thermal performance framing, in which every material is applied at the thickness required to meet a common thermal transmittance target, and implicitly assume that the resulting ranking varies with climate. The purpose of this study is not to identify a best material, but to test that framing itself within the six degree-day zones introduced by TS 825:2024, which became mandatory in Türkiye on 1 April 2025 and, for the first time, incorporates cooling demand alongside heating. Theory and Methods: Fourteen bio-based and four conventional alternatives were compared using a functional unit of one square metre of external wall meeting the recommended transmittance of the relevant zone. Criteria comprised thermal conductivity, unit mass, areal heat capacity, biogenic carbon, reaction to fire, end-of-life scenario, moisture sensitivity and required insulation thickness. Weights were derived by Shannon entropy, by CRITIC and by equal weighting; rankings by TOPSIS, with VIKOR used as a complementary ranking method for methodological sensitivity. Environmental data were life cycle assessment datasets obtained from OEKOBAUDAT, whose dataset identifiers, versions and reference units are reported; climate data were taken from version 5.3 of the open European PVGIS service, using the 2005-2023 long-term means. The calculation basis was cross-checked against a published minimum insulation thickness table at eight data points, with exact agreement. Missing values were not set to zero; only criteria with complete data across all alternatives entered the main run, and the sensitivity of this rule to the threshold was tested. Because the reaction-to-fire classification is ordinal, three numerical encodings of it were compared. Three system boundaries were defined from the declaration modules, and the realised end-of-life release fraction was treated as a sensitivity parameter. Results: The required insulation thickness equals the thermal conductivity multiplied by a factor that depends only on the zone, so every criterion derived from thickness carries a common, material-independent factor that cancels exactly in any normalisation insensitive to column-wise scaling. With rounding disabled, the inter-zone rank correlation is 1.000000 for three weighting methods and two ranking methods, and the maximum difference between weights across zones is of the order of 1e-9, that is, numerically zero. A zone effect emerges only when structures that break this proportionality are introduced: a buildability constraint eliminates six bio-based alternatives in the coldest zones and reduces the correlation from 0.990 to 0.497, while climate-dependent weighting reduces it to 0.513. Entropy assigns approximately one third of the total weight to reaction to fire, on which mineral-based products separate sharply from the rest, and therefore pulls mineral wools forward, whereas CRITIC and equal weighting recognise the same criterion without letting it dominate. This behaviour is sensitive to how the ordinal class is encoded: a binary encoding raises the entropy weight to 38.5 per cent and lowers the rank correlation with the baseline encoding to 0.453, while CRITIC and equal weighting remain at 0.744 and 0.903. Agreement between TOPSIS and VIKOR is 0.352 to 0.633. Product declaration modules show that the biogenic carbon taken up in production is released again at end of life; for six materials uptake and release cancel to within 0.04 kgCO2e/kg. Truncating the account at the factory gate makes bio-based materials appear carbon-negative and places them first, whereas including end-of-life modules moves glass wool to first place in every zone examined. A bio-based material remains first until more than half of the stored carbon is released. Conclusion: The three results converge on a single conclusion: a material ranking is interpretable only when the framing that produced it, namely the functional unit, the weighting method, the representation of the criteria, the ranking method and the life cycle system boundary, is fully declared. A ranking is not an intrinsic property of the materials. Two consequences follow for practice. Under a recommended transmittance requirement, the degree-day zone alone does not differentiate material choice; differentiation by zone arrives through the buildability constraint. The monthly quasi-steady formulation adopted here has limited sensitivity to thermal-mass effects compared with an hourly dynamic approach such as that provided by ISO 52016-1. The rankings reported here are indicative and remain sensitive to data quality and criterion representation, whereas the analytical invariance result is derived from the framing itself. |
| Keywords: |  |  |
| - Bio-based insulation - Multi-criteria decision making - Life cycle system boundary - TS 825 - Weighting |  |  |
| Article Info: |  |  |
| Received: dd.mm.yy Accepted: dd.mm.yy DOI: |  |  |
| Acknowledgement: |  |  |
| Correspondence: |  |  |
| Author: e-mail: phone: |  |  |
| YAZARA NOT: Dergimizin Uluslararası Görünürlüğü ve Makalelerin yurtdışından atıf almalarının en önemli özelliği Extenden Abstract kısmı olup; Siz değerlerli yazarların, Genişletilmiş İngilizce Özet sayfasını ve Figure A resmini bu Dosyada belirtilen formatta hazırlaması büyük önem arzetmektedir. Böylelikle yabancı yazarların ve okuyucuların sizin makalenizde yer alan önemli bilgilerden faydalanabilmeli ve atıf yapmalarına kolaylık sağlamalıdır. |  |  |